# Boiler vs Furnace: Operations Decision Brief for Adatum

Prepared for C-suite review: energy efficiency, total cost, HVAC impact, maintenance, and reliability

# Executive Summary

สำหรับ Adatum การเลือกระหว่าง Boiler และ Furnace ไม่ควรตัดสินจากค่า efficiency ของเครื่องเพียงอย่างเดียว เพราะ AFUE ไม่รวม distribution loss จาก duct หรือ piping ซึ่งอาจมีผลต่อต้นทุนจริงและประสิทธิภาพรวมของอาคารอย่างมีนัยสำคัญ ข้อมูลจาก U.S. Department of Energy ระบุว่า high-efficiency furnace และ boiler อยู่ในช่วงประมาณ 90%–98.5% AFUE แต่ระบบ forced-air furnace อาจสูญเสียพลังงานผ่าน duct ได้มาก โดยเฉพาะเมื่อ duct อยู่ในพื้นที่ไม่ปรับอากาศ ขณะที่ boiler มีข้อได้เปรียบด้าน radiant/hydronic comfort และอายุการใช้งานโดยรวม แต่มี CAPEX และความซับซ้อนด้าน piping, water treatment และ compliance สูงกว่า

Recommendation: เลือก Furnace เมื่อ Adatum ต้องการ lowest upfront cost, retrofit เร็ว, ใช้ ductwork เดิม, และต้องการผสาน heating/cooling ใน HVAC เดียวกัน เลือก Boiler เมื่ออาคารหรือโรงงานต้องการ process heat, radiant heating, zoning ที่ละเอียด, comfort/quiet operation สูง, หรือมี infrastructure hydronic/steam อยู่แล้ว สำหรับโรงงานที่มีโหลดความร้อนต่อเนื่องและต้องการ reliability สูง ควรพิจารณา Boiler พร้อม redundancy และ preventive maintenance program; สำหรับอาคารสำนักงานทั่วไปที่มี ducted cooling อยู่แล้ว Furnace หรือ packaged rooftop/air-handling solution มักเหมาะกว่าในเชิง CAPEX และ implementation speed

# Comparison Table

| Decision Area | Boiler | Furnace | Operations Implication for Adatum |
| --- | --- | --- | --- |
| Energy efficiency | Modern condensing boilers can achieve high AFUE; hot-water systems are generally more efficient than steam systems because steam operates at higher temperatures. | Modern condensing furnaces can also reach high AFUE; DOE standards for residential gas furnaces move toward 95% AFUE for new products in late 2028. | Do not compare AFUE alone. Evaluate whole-system efficiency including duct leakage, pipe insulation, controls, zoning, fan/pump energy, and operating profile. |
| CAPEX | Typically higher where new piping, radiators, condensate handling, water treatment, or boiler room upgrades are required. | Typically lower when ductwork and gas/electrical service already exist; faster retrofit path for office buildings. | Furnace is often preferable for budget-constrained building retrofit; Boiler is justified when hydronic infrastructure or process heat requirements exist. |
| OPEX | Can be lower for steady loads, zoning, radiant heat, and hydronic applications; requires water treatment and inspections. | Fuel OPEX can be competitive with high-efficiency equipment, but fan power, filter replacement, and duct losses can increase real operating cost. | Model OPEX using local fuel price, load duration curve, utilization hours, and maintenance staffing—not generic averages. |
| Maintenance requirements | Requires combustion checks, burner service, pumps/valves, pressure controls, water chemistry, controls calibration, and safety inspections. | Requires filter changes, blower/heat exchanger inspection, combustion checks, ignition/sensor maintenance, belt/motor service, and duct cleanliness. | Boiler maintenance is more specialized and compliance-heavy; Furnace maintenance is simpler but more frequent at distributed units. |
| HVAC impact | Hydronic/radiant distribution decouples heating from ventilation and cooling; may require separate AHU/DOAS for ventilation and cooling. | Forced-air heating integrates naturally with cooling coils, filtration, humidification, and ventilation through ducts. | If Adatum prioritizes integrated HVAC and indoor air quality upgrades, Furnace/forced-air architecture is simpler. If comfort and zoning dominate, Boiler/hydronic is stronger. |
| Failure rate / reliability | ASHRAE service-life data indicates boilers can have long median service lives, with hot-water/steam boiler categories commonly ranging around mid-20s to 30+ years depending on construction. | ASHRAE life expectancy references commonly indicate gas/oil-fired furnaces around 18 years median service life. | Boiler plants can be highly reliable when designed with redundancy and maintained well, but single-point failures can be high impact. Furnace systems are easier to modularize but may create many distributed maintenance points. |

# Operational Advantages and Disadvantages

## Boiler

- ข้อดี: เหมาะกับโหลดความร้อนต่อเนื่อง, radiant comfort, zoning ละเอียด, noise ต่ำ, และอายุการใช้งานยาวเมื่อดูแลถูกต้อง

- ข้อเสีย: CAPEX สูงกว่าใน retrofit ที่ไม่มี piping, ต้องใช้ช่างเฉพาะทาง, มีความเสี่ยงจาก pressure, water chemistry, corrosion, scaling, leakage และ downtime ของ central plant

- เหมาะกับ: โรงงาน, process heat, อาคารที่มี hydronic หรือ steam infrastructure, พื้นที่ต้องการ comfort สูง, หรือพื้นที่ที่ต้องการแยก heating ออกจาก ventilation/cooling

## Furnace

- ข้อดี: CAPEX ต่ำกว่าเมื่อมี ductwork เดิม, ติดตั้งเร็ว, รวม heating/cooling/filtration/ventilation ในระบบ forced-air ได้ง่าย, และบำรุงรักษาเข้าใจง่าย

- ข้อเสีย: duct leakage และ fan energy อาจลด real-world efficiency, มี filter และ airflow maintenance ต่อเนื่อง, อาจเกิด thermal stratification, noise, draft และการกระจายฝุ่นหาก duct/filter ไม่ดี

- เหมาะกับ: อาคารสำนักงาน, warehouse ที่ใช้ forced-air อยู่แล้ว, retrofit ที่ต้องการ speed และ cost control, หรือพื้นที่ที่ cooling และ ventilation เป็น priority หลัก

# Key Risks and Mitigations

| Risk | Higher Exposure | Mitigation |
| --- | --- | --- |
| Energy savings not realized | Both | Commissioning, right-sizing, controls tuning, pipe/duct insulation, duct leakage testing, and measurement & verification. |
| Single-point plant failure | Boiler | N+1 boiler capacity, redundant pumps, spare critical controls, water treatment program, and emergency bypass plan. |
| High distributed maintenance workload | Furnace | Standardize equipment models, maintain filter replacement schedule, remote monitoring, and planned seasonal inspections. |
| Indoor air quality degradation | Furnace | Upgrade filtration, maintain duct cleanliness, ensure ventilation rates, and balance airflow. |
| Water leakage, corrosion, scaling | Boiler | Water treatment, periodic inspection, corrosion monitoring, leak detection, and preventive replacement of valves and pumps. |

# Decision Guidance for Adatum

- Choose Boiler for factories or buildings with continuous thermal loads, process heat needs, existing steam/hydronic networks, high comfort requirements, or where long asset life and zoning justify higher CAPEX.

- Choose Furnace for office buildings or light-industrial spaces where ductwork already exists, cooling and ventilation integration is important, project timeline is short, and upfront capital discipline is the primary constraint.

- Use lifecycle-cost analysis before final approval: compare 15–25 year TCO, local fuel/electricity rates, maintenance labor model, expected utilization hours, redundancy requirement, and downtime cost per hour.

- For mixed campuses, use a hybrid strategy: boiler plant for production/process or high-comfort zones; furnace/forced-air or rooftop units for offices, warehouses, and zones needing integrated cooling and ventilation.

# Public Sources Used

- U.S. Department of Energy, “Furnaces and Boilers”: definitions of furnace vs boiler, AFUE, efficiency ranges, and note that AFUE excludes duct/piping losses.

- U.S. Department of Energy, 2023 residential furnace efficiency standards announcement: upcoming 95% AFUE standard for residential gas furnaces manufactured in late 2028 and projected energy-bill savings.

- ENERGY STAR, “Furnaces”: efficiency benefits of ENERGY STAR certified furnaces and expected regional savings.

- ASHRAE Standards and Guidelines: references for HVAC inspection, maintenance, load calculations, and system guidance, including Standard 180 for inspection and maintenance of commercial HVAC systems.

- ASHRAE HVAC Service Life and Maintenance Cost Database: public database for HVAC equipment service life and maintenance cost benchmarking.